School of Electronic

and Electrical Engineering

ELEC5566M: FPGA Design for System on Chip

ASSIGNMENT 1
N-Channel 8-bit Servo Controller



STUDENT NAME: SHRAJAN BHANDARY
STUDENT ID       :  201289634

N-CHANNEL 8-BIT SERVO CONTROLLER

1. ABSTRACT

The project describes a procedure to develop an IP core for N-channel 8-bit Servo Controller using Behavioural Verilog HDL. The N-channel 8-bit servo controller was built with a hierarchical approach by instantiating multiple low-level entities as submodules. The top-level parametrised servo controller module was tested and verified using both test-benches and hardware implementation.

2. INTRODUCTION

Servomotors are linear or rotary actuators that render accurate control of linear or angular positions. The motors are embedded with feedback sensors to control the positions. Dedicated controller circuits are required to achieve desired positions [1]. The servomotors used in the project are of the type SG90 and each of them consists of a DC motor operating at 5V. In addition, the shaft of the motor is coupled to a potentiometer to retrieve the position and control the actuation. The inputs to these servomotors are Pulse Width Modulated (PWM) signals and the signals are usually generated by a controller with the aid of a clock signal. SG90 servomotors can actuate to angular positions ranging from -90 degrees (0 degrees) to +90 degrees (180 degrees). In order to control the servomotor, the period of the PWM signals must be a fixed value of 20ms. The minimum ON time should be 0.5ms to actuate to an angular position of -90 degrees and the maximum ON time should be 2.5ms to actuate to an angular position of +90 degrees. The required PWM input signal to the servomotor along with the different ON periods and their corresponding angular positions is depicted in figure 1.

Figure 1: Controller output PWM signals – ON Time vs. Angular Position.

To generate the PWM signals and develop the N-channel 8-bit servo controller, the on-board clock of the Cyclone V FPGA system, oscillating at 50 MHz, must be utilized. The ON time can be varied by the manipulating the 8-bit input signals connected to 8 different slide switches of the FPGA board. The positions of the connected servomotors are individually loaded onto their corresponding PWM ports using the address selector and the latch signal which are connected to 2 slide switches and 1 push button of the Cyclone V board, respectively. The N-channel 8-bit servo controller can also be simulated using test benches with a clock frequency varying from 128 kHz to 100 MHz.

3. THE PROPOSED CONTROLLER

The N-channel 8-bit Servo Controller was developed by instantiating multiple submodules. These low-level entities perform different functions to produce the desired PWM signals. The submodules used are: N-bit counter, N-bit comparator, single servo controller (with fixed clock frequency) and frequency divider.

3.1. CALCULATING THE REQUIRED CLOCK CYCLES

Before delineating about the submodules, it is crucial to generate the exact period of the PWM signal. This can be achieved by implementing the controller circuit with a fixed input clock having a frequency of 128 kHz. The time period of each cycle generated by this clock circuit is 7.8125us. The number of clock cycles required to generate one complete PWM signal of period 20ms is:

20 ms7.8125 us = 2560 cycles…………………………………………… (1)

The number of clock cycles required to generate the minimum ON time of 0.5ms is:

0.5 ms7.8125 us = 64 cycles…………………………………………… (2)

The number of clock cycles required to generate the maximum ON time of 2.5ms is:

2.5 ms7.8125 us = 320 cycles…………………………………………… (3)

3.2. SUBMODULES

The two major submodules of the N-channel 8-bit Servo Controller are: the module that controls a single servo with a fixed frequency and the module that converts any incoming clock frequency to a fixed clock rate. These modules instantiate submodules of their own to perform the necessary operations.

3.2.1. Single Servo Controller Submodule

The module is capable of controlling a single servo using an 8-bit input and a fixed clock frequency of 128kHz. The first task of the module is to count the number of the cycles derived in equations 1, 2 and 3. This is achieved by instantiating a submodule called N-bit counter with a maximum count value of 2560 cycles and supply it with the clock. When the maximum value is reached, the counter value is reset to 0. The second task is to calculate the threshold clock cycle value (shown below in equation 4) where the PWM signal of the servo motor is HIGH. The equation for the threshold is given by:

THRESHOLD = 8-bit INPUT + 64……….………….…………(4)

The final task is to compare the threshold value with the generated clock cycles and then switch the PWM signal accordingly. The reset input actuates the servomotor to its default angular position of 0 degrees. The load input latches the value of the 8-bit duty cycle control onto the PWM output of the module. The implementation of the submodules is described in Appendix A – Section 1, 2 and 3. In order to verify the module, a test bench was developed. The test bench simulates an input clock signal with frequency of 128 kHz, a load signal, a reset signal and an 8-bit duty cycle control that changes its values after repeating for a discrete value of time.

Figure 2: Input and Output Waveforms of submodule single servomotor controller with 128 kHz clock using ModelSim.

It is evident from figure 2 that for different values of the duty cycle control (8-bit input), the PWM signals have different ON-time values with a fixed clock period. This submodule, as a standalone, is incapable of controlling a servomotor with the Cyclone V FPGA board, since the on-board clock has a frequency of 50 MHz.

3.2.2. Frequency Divider Circuit Submodule

The limitation of the single servomotor controller is the fact that it can only be implemented on a board with a 128 kHz clock frequency. This drawback can be overcome by designing a circuit that can convert any incoming clock frequency to the required value. The maximum frequency that could be injected into the module is 100 MHz and the minimum frequency is 128 kHz. The frequency divider circuit takes any incoming signal and divides it by the required frequency to obtain a factor called the toggle value. An N-bit counter is instantiated at this stage using a maximum counter limit equal to the toggle value. The output of the counter could then be used to generate an output clock signal that switches (LOW to HIGH or vice-versa) at every instance the maximum counter value is reached. However, the frequency of the resultant signal would be half the value as that of the required clock signal. The reason is that toggle value consists of both the ON cycles and the OFF cycles [2-3]. Therefore, dividing the toggle value by a factor of 2 will produce the desired fixed clock frequency of 128 kHz. The toggle value is given by:

Toggle value = Incoming frequency2 * Required Fixed frequency ……………………………… (5)

Figure 3: Input and Output Waveforms of submodule frequency divider with input clock of 50 MHz clock using ModelSim.

The test bench simulates an input clock signal with frequency of 50 MHz and a reset signal. Figure 3 illustrates the working of the frequency divider with an input clock signal of 50 MHz frequency and the resultant output clock signal of 128 kHz frequency. The submodule is illustrated in Appendix A – Section 4.

3.3. SERVO CONTROLLER WITH 50 MHz CLOCK

The single servo controller with a parameterised clock frequency of 50 MHz was developed to test the hardware using the SG90 motor and Cyclone V FPGA board. In addition, this module was also tested using a test bench. The top-level entity is parameterised such that the input clock frequency can be changed. This module then instantiates the two submodules: servo controller with fixed clock frequency of 128 kHz and the frequency divider circuit module. The input clock signal is connected to the frequency divider module that outputs a fixed clock rate. This output clock signal is then supplied to the servo controller module that generates the PWM signal. The module is illustrated in Appendix A – Section 5. This module was tested using ModelSim by creating a test bench having a clock frequency of 50 MHz. The input duty cycle control and the load signals were changed after every 20ms to produce different PWM signals as shown in figure 4. The hardware verification was carried out with the input clock of the module connected to the 50 MHz of the FPGA board. The input duty cycle control was connected to the slide switches. The load signal and the reset signal were connected to two separate push buttons. However, the push buttons are active low and hence the signals were inverted in the module. The Verilog module is illustrated in Appendix A – Section 5.

Figure 4: Input and Output Waveforms of the single servo motor using a frequency divider circuit.

3.4. N-CHANNEL 8-BIT SERVO CONTROLLER

The single servo test module implemented using 50 MHz input clock rate is hardwired to drive only one servomotor. However, the objective of the project is to develop an IP core that can render and utilize N independent channels. Therefore, the top-level module of the N-channel controller instantiates a parameterised number of servo addresses. This module has a parameterised clock input, a single 8-bit input to set the duty cycle, a load signal to latch the duty cycle value, an address input to update individual servos at a given instance and an N-bit PWM output. Since the number of channels is arbitrary, the servo selector address input must not have a fixed width. This is executed using a pre-processor macro - ceil(log2(N)) as described in Appendix A – section 6. Similar to the module described in section 3.3, the N-channel servo controller module also instantiates the frequency divider circuit. The next task is to initialize N number of servo controller module (section 3.2.1). This is accomplished by a ‘for-loop’ used within an advanced parametrisation block called ‘generate’. The for-loop iterates for values ranging from 0 to (N-1) and instantiates N number of servo controller modules. Furthermore, each instantiated module declared within the for-loop will have the same ports connected to them except for the PWM and load signals. The individual load signals, which control the PWM signals, are determined by the combination of values of the address input and latch signal as shown in figure 5.

Figure 5: Instantiating ‘N’ modules and selecting the servo to be updated (Appendix A -Section 6).

A test bench was created for this module, similar to the one outlined in section 3.3, that simulates 4 different servo addresses, a 1-bit load signal and an 8-bit duty cycle control with an input clock signal of 50 MHz. The output waveforms depict 4 different values of PWM, each having different ON times with a fixed clock period as shown in Figure 6. This module was also tested using hardware verification by employing 3 servomotors as shown in figure 7. The Verilog module is described in Appendix A – Section 6.

Figure 6: Input and Output Waveforms of the N-channel 8-bit servo controller using ModelSim.

Figure 6: Hardware implantation of N-channel 8-bit servo controller.

4. CONCLUSIONS

The project successfully establishes an N-channel 8-bit Servo Controller that can work with clock frequencies from 128kHz to 100MHz. The hierarchical structure that represents the complex module of the controller permits easy debugging of errors. Furthermore, simulation using test benches helps in understanding both the flow of the Verilog description and also any shortcomings that exist in the module.

5. REFERNCES

- Wikipedia. Servomotor. [Online]. 2019. [Accessed 10 March 2019]. Available from: https://en.wikipedia.org/wiki/Servomotor

- Ma, S., Guan, B., and Hou, L. Design of high-accuracy decimal frequency divider with Verilog-HDL. In: International Power, Electronics and Materials Engineering Conference, 2015, China. doi: 10.2991/ipemec-15.2015.16

- Zhang, S. W., and Zhao, C. Design for realizing arbitrary fractional divider based on FPGA which duty cycle is up to 50%. Proceedings of the 2nd International Symposium on Computer, Communication, Control and Automation. Singapore. pp. 1653-1657, 2013.

APPENDIX – A

Appendix - A consists of all the Verilog module files used in the project.

SECTION 1: N-bit counter module

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Verilog Module Name : N_Bit_Counter Code Author : Shrajan Bhandary Date Created : 26/02/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to count values ranging from 0 to required maximum value. Once the maximum value is reached the counter is reset to 0. The counter increments at every positive edge of clock signal and can be reset at every positive edge of the reset signal i.e., when the reset button is pressed. For this project the maximum value corresponds to the clock period (20 ms) of the PWM Signal of the Servo Motor. */ //////////////////////////////////////////////////////////////////////////////// module N_Bit_Counter #( // Start of the module. /* Parameter List of the N_Bit_Counter */ parameter COUNTER_VALUE_WIDTH = 12, // The default width is 12 bits. parameter COUNTER_MAX_VALUE = (2**COUNTER_VALUE_WIDTH)-1, // The maximum value that corresponds to (2^width - 1). parameter COUNTER_INCREMENT = 1 // The counter should increment by 1 every clock cycle. )( /* Port List of the N_Bit_Counter */ input COUNTER_CLOCK , // Counter increments according to the clock. input COUNTER_RESET , // Counter resets to 0 when reset becomes HIGH. input COUNTER_ENABLE , // Counter increments only if enable is HIGH. output reg [(COUNTER_VALUE_WIDTH-1):0] COUNTER_VALUE = 0 // The final count value of the counter. ); /* Local Parameter List of the N_Bit_Counter */ localparam ZERO = {(COUNTER_VALUE_WIDTH){1'b0}}; // Local parameter with value 0 having default width of 12 bits. always @ ( posedge COUNTER_CLOCK or posedge COUNTER_RESET ) // Always statement such that the counter value changes when either begin // reset or clock change from LOW to HIGH. if ( COUNTER_RESET ) // Check whether reset is HIGH. begin COUNTER_VALUE <= ZERO; // Set counter value to 0 if reset is HIGH. end else if ( COUNTER_ENABLE ) // Check if enable is HIGH begin if ( COUNTER_VALUE >= COUNTER_MAX_VALUE ) // Check if counter value has surpassed maximum value. begin COUNTER_VALUE <= ZERO; // Set counter value to 0 if counter value has surpassed maximum value. end else begin COUNTER_VALUE <= COUNTER_VALUE + COUNTER_INCREMENT; // If none of the above conditions satisfy, then increment the counter value. end end end endmodule // End of the module.

SECTION 2: N-bit comparator module

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : N_Bit_Comparator Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 26/02/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used compare the value of two numbers. The output of the module is HIGH when the first number is greater than or equal to the second number. The output of the module is LOW when the first number is smaller than the second number. The time period for which the output of the comparator is HIGH determines the ON_PERIOD of the PWM Signal of the Servo Motor. */ //////////////////////////////////////////////////////////////////////////////// module N_Bit_Comparator #( // Start of the module. /* Parameter List of the N_Bit_Comparator */ parameter NUMBER_WIDTH = 12 // The default width is 12 bits to match value from the counter. )( /* Port List of the N_Bit_Comparator */ input [(NUMBER_WIDTH-1):0] FIRST_NUMBER , // The first number of the comparator. input [(NUMBER_WIDTH-1):0] SECOND_NUMBER , // The second number of the comparator. output reg FN_GREATER_THAN_SN // The output of the comparator. ); always @ ( FIRST_NUMBER , SECOND_NUMBER ) begin if ( FIRST_NUMBER >= SECOND_NUMBER ) // LOW if second number is greater than first number, else HIGH. begin FN_GREATER_THAN_SN <= 1'b1; end else begin FN_GREATER_THAN_SN <= 1'b0; end end endmodule // End of the module.

SECTION 3: Servo Motor Controller Module

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : Servo_Motor_Controller Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to control a single SG90 Servo Motor. The servo motor requires an input PWM signal to drive to the required angular position. The PWM signal for the SG90 motor should have a clock period of 20 ms. The desired angular position can be obtained by controlling the ON period (duty cycle) of the PWM signal. The duty cycle is can be manually varied by changing the 8 bit inputs connected to 8 different slide switches. The value of the duty cycle is latched to the output by using a load signal connected to a push button. The servo motor can be initialised to a default position by pressing the reset push button. The default frequency of the clock is 128 kHz. */ //////////////////////////////////////////////////////////////////////////////// module Servo_Motor_Controller # ( // Start of the module. /* Parameter List of the Servo_Motor_Controller */ parameter DUTY_CYCLE_WIDTH = 8 , // The default width is 8-bits ( 0 to 255 ) corresponding to ( -90 to +90 ). parameter PWM_SIGNAL_WIDTH = 12 , // The required width is 12 bits to enclose maximum value of 2560 (20ms) with a clock of 128 kHz frequency. parameter MAX_CLOCK_CYCLES = 2560 // With a clock of 128 kHz frequency, 2560 clock cycles are required to generate 20 ms. )( /* Port List of the Servo_Motor_Controller */ input CLOCK_SIGNAL , // Connects to the hardware clock to generate the required PWM signal. input CLOCK_ENABLE , // Servo motor turns only if enable is HIGH. input SERVO_RESET , // Servo resets to default when reset becomes HIGH. input LOAD_SIGNAL , // Latch the duty cycle of the PWM signal. input [(DUTY_CYCLE_WIDTH-1):0] DUTY_CYCLE_CONTROL, // Controls the duty cycle of the PWM signal, thus controlling the angle. output PWM_SIGNAL // The output of the module that is the input to the Servo motor. ); /* Register List of the Servo_Motor_Controller */ wire [(PWM_SIGNAL_WIDTH-1):0] CURRENT_CLOCK_CYCLES; // This holds the number of clock cycles generated by the clock with 128 kHz frequency. reg [(PWM_SIGNAL_WIDTH-1):0] REQUIRED_ON_CYCLES ; // The ON period (duty cycle) is determined by the 8-bit input. wire DIVIDED_CLOCK_SIGNAL; // Output of the frequency divider circuit that converts current clock frequency to the default value. /* Local Parameter List of the Servo_Motor_Controller */ localparam DEFAULT_ON_CYCLES = {(PWM_SIGNAL_WIDTH){12'd192}}; // To set the servo to angle 0, 1.5ms is required which corresponds to 192 clock cycles. localparam OFFSET_ON_CYCLES = {(PWM_SIGNAL_WIDTH){12'd64}} ; // The on cycles vary from 64(0.5ms) to 320(2.5ms) to turn angles from -90 to 90. /* Check status of load signal and rest and then take necessary actions */ always @ ( posedge LOAD_SIGNAL or posedge SERVO_RESET ) // Always statement is used to set the required on cycle depending on the value of the 8-bit input. begin // The sensitivity list contains the latch signal and reset signal. if ( SERVO_RESET ) // Check if reset is HIGH. begin REQUIRED_ON_CYCLES <= DEFAULT_ON_CYCLES; // Set the servo to its default position if the reset if HIGH. end else if ( CLOCK_ENABLE ) // Check if clock enable is HIGH. begin REQUIRED_ON_CYCLES <= DUTY_CYCLE_CONTROL + OFFSET_ON_CYCLES;// Assigning value to the on cycle based upon the duty cycle and the offset. end end /* Instantiating N Bit Counter to generate the required clock cycles for the servo motor. */ N_Bit_Counter # ( . COUNTER_VALUE_WIDTH ( PWM_SIGNAL_WIDTH ), // Setting the parameter values. . COUNTER_MAX_VALUE ( MAX_CLOCK_CYCLES ) ) Cycle_Generator ( . COUNTER_CLOCK ( CLOCK_SIGNAL ), // Setting the connections to their corresponding ports. . COUNTER_RESET ( SERVO_RESET ), . COUNTER_ENABLE ( CLOCK_ENABLE ), . COUNTER_VALUE ( CURRENT_CLOCK_CYCLES ) ); /* Instantiating N Bit Comparator to generate the HIGH cycles of the PWM signal. */ N_Bit_Comparator # ( . NUMBER_WIDTH ( PWM_SIGNAL_WIDTH ) // Setting the parameter values. ) On_Period_Generator ( . FIRST_NUMBER ( REQUIRED_ON_CYCLES ), // Setting the connections to their corresponding ports. . SECOND_NUMBER ( CURRENT_CLOCK_CYCLES ), . FN_GREATER_THAN_SN ( PWM_SIGNAL ) ); endmodule // End of the module.

SECTION 4: Frequency divider circuit module

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : Frequency_Divider Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to reduce the incoming clock rate to a fixed value. The fixed value is 128 kHz and the incoming clock rate can vary from 128 kHz to 100 MHz. */ //////////////////////////////////////////////////////////////////////////////// module Frequency_Divider #( // Start of the module. /* Parameter List of the Frequency_Divider */ parameter INCOMING_CLOCK_FREQUENCY = 50000000, // The minimum operable frequency is 128 kHz and the maximum operable frequency is 100 MHz. parameter FIXED_CLOCK_FREQUENCY = 128000, // All the servo default parameters are calculated at this frequency. parameter TOGGLE = (INCOMING_CLOCK_FREQUENCY / FIXED_CLOCK_FREQUENCY)/2, // The toggle value is used to change the state of the output. The maximum value is 781. parameter MAXIMUM_WIDTH = 10 , // 10 bits to encompass all the possible values of toggle. parameter INCREMENT = 1 // The value by which the clock counter should increase. )( /* Port List of the Servo_Motor_Controller */ input FD_CLOCK_IN, // The incoming clock is connected to this port. input FD_RESET, // The reset pin is connected to this port. output reg FD_CLOCK_OUT // This provides the fixed clock rate depending upon the divider value. ); wire [(MAXIMUM_WIDTH-1):0] CURRENT_COUNTER ; // This maximum value of the counter will be equal to the toggle which needs 10 bits. localparam LOW = 0; // 1 bit Local parameter with value 0 localparam HIGH = 1; // 1 bit Local parameter with value 1 /* Instantiating the N Bit counter to count values up to Toggle */ N_Bit_Counter # ( . COUNTER_VALUE_WIDTH ( MAXIMUM_WIDTH ), . COUNTER_MAX_VALUE ( TOGGLE ), . COUNTER_INCREMENT ( INCREMENT ) ) FD_Toggler ( . COUNTER_CLOCK ( FD_CLOCK_IN ), . COUNTER_RESET ( FD_RESET ), . COUNTER_ENABLE ( HIGH ), . COUNTER_VALUE ( CURRENT_COUNTER ) ); always @ ( posedge FD_CLOCK_IN or posedge FD_RESET ) // Always statement such that the counter value changes when either begin // reset or clock change from LOW to HIGH. if ( FD_RESET ) // Check whether reset is HIGH. begin FD_CLOCK_OUT <= LOW; // Reset the initialize the clock. end else if ( CURRENT_COUNTER == TOGGLE ) // Check if the toggle value has been reached. begin FD_CLOCK_OUT <= ~ FD_CLOCK_OUT; // Switch (LOW to HIGH or HIGH to LOW ) the output clock when the current value has reached the toggle value. end end endmodule // End of the module

SECTION 5: Test single servo motor module

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Verilog Module Name : Test_Single_Servo_Motor Code Author : Shrajan Bhandary Date Created : 03/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to control a single SG90 Servo Motor. The servo motor requires an input PWM signal to drive to the required angular position. The PWM signal for the SG90 motor should have a clock period of 20 ms. The desired angular position can be obtained by controlling the ON period (duty cycle) of the PWM signal. The duty cycle is can be manually varied by changing the 8 bit inputs connected to 8 different slide switches. The value of the duty cycle is latched to the output by using a load signal connected to a push button. The servo motor can be initialised to a default position by pressing the reset push button. */ //////////////////////////////////////////////////////////////////////////////// module Test_Single_Servo_Motor #( // Start of the module. /* Parameter List of the Test_Single_Servo_Motor */ parameter CLOCK_FREQUENCY = 50000000, // The minimum operable frequency is 128 kHz and the maximum operable frequency is 100 MHz. ( In Hz). parameter DUTY_CYCLE_WIDTH = 8 , // The number of bits for the duty cycle control. parameter INVERTED_INPUT = 1 // Parameter to select between active LOW ( Invert = 1 ) inputs and active HIGH ( Invert = 0 )inputs. // Inverted input should be 0 and 1 for test bench verification and hardware verification respectively. )( /* Port List of the Test_Single_Servo_Motor */ input CLOCK , // The incoming clock is connected to this port. input RESET , // The reset pin is connected to this port. input LOAD_SIGNAL , // Latch the duty cycle of the PWM signal. input [(DUTY_CYCLE_WIDTH-1):0] DUTY_CYCLE_CONTROL, // Controls the duty cycle of the PWM signal, thus controlling the angle. // output DIVIDED_CLOCK , // This signal is used only in test bench verification to check the divided clock. // Comment lines 40 and 49 to perform hardware verification. output PWM_SIGNAL // The output of the module that is the input to the Servo motor. ); wire CLOCK_CONNECTOR; // Net to connect output of the frequency divider to the servo motor controller. reg INVERT_RESET; // Inverts the input reset signal for LOW active low inputs. reg INVERT_LOAD_SIGNAL; // Inverts the input load signal for LOW active low inputs. localparam HIGH_VALUE = 1; // Local parameter with value 1. // assign DIVIDED_CLOCK = CLOCK_CONNECTOR; // This signal is used only in test bench verification to check the divided clock. Comment this for hardware verification. /* To check whether the inputs are active LOW or not and then take necessary actions */ always @ ( RESET , LOAD_SIGNAL ) begin // Always statement that changes with respect to reset and load signals. if ( INVERTED_INPUT ) // Check if the inputs are active LOW or active HIGH. begin // If yes, invert the inputs. INVERT_RESET <= ~ RESET; INVERT_LOAD_SIGNAL <= ~ LOAD_SIGNAL; end else begin // If no, assign the same inputs. INVERT_RESET <= RESET; INVERT_LOAD_SIGNAL <= LOAD_SIGNAL; end end /* Instantiating the frequency divider to convert incoming clock frequency to fixed frequency. */ Frequency_Divider # ( . INCOMING_CLOCK_FREQUENCY ( CLOCK_FREQUENCY ) ) Clock_Enable_Generator ( . FD_CLOCK_IN ( CLOCK ), . FD_RESET ( INVERT_RESET ), . FD_CLOCK_OUT ( CLOCK_CONNECTOR ) ); // Instantiating N number of servomotors with separate PWM signal and load signal. Servo_Motor_Controller Servo_Motor_Driver_Block ( . CLOCK_SIGNAL ( CLOCK_CONNECTOR ), . CLOCK_ENABLE ( HIGH_VALUE ), . SERVO_RESET ( INVERT_RESET ), . LOAD_SIGNAL ( INVERT_LOAD_SIGNAL ), . DUTY_CYCLE_CONTROL ( DUTY_CYCLE_CONTROL ), . PWM_SIGNAL ( PWM_SIGNAL ) ); endmodule

SECTION 6: N-channel 8-bit servo controller

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : N_Channel_Servo_Controller Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to instantiate a parametrized number of servo controllers to allow the IP core to have N independent channels. This module has a clock input, an N-bit PWM output, a single 8-bit input to set the duty cycle, a 1-bit load signal to latch the duty cycle value, and an address input to control which servo is being updated. */ //////////////////////////////////////////////////////////////////////////////// /* ceil(log2(N)) Preprocessor Macro */ `define clog2(x) ( \ ((x) <= 2) ? 1 : \ ((x) <= 4) ? 2 : \ ((x) <= 8) ? 3 : \ ((x) <= 16) ? 4 : \ ((x) <= 32) ? 5 : \ ((x) <= 64) ? 6 : \ ((x) <= 128) ? 7 : \ ((x) <= 256) ? 8 : \ ((x) <= 512) ? 9 : \ ((x) <= 1024) ? 10 : \ ((x) <= 2048) ? 11 : \ ((x) <= 4096) ? 12 : 16) module N_Channel_Servo_Controller #( // Start of the module. /* Parameter List of the N_Channel_Servo_Controller */ parameter NO_OF_CHANNEL = 4 , // The number of servos to be controlled corresponding to number of PWM signals. parameter ADDRESS_WIDTH = `clog2(NO_OF_CHANNEL), // Determine the required width to of the servo selector. parameter CLOCK_FREQUENCY = 50000000 , // The minimum operable frequency is 128 kHz and the maximum operable frequency is 100 MHz. ( In Hz). parameter DUTY_CYCLE_WIDTH = 8 , // The number of bits for the duty cycle control. parameter INVERTED_INPUT = 1 // Parameter to select between active LOW ( Invert = 1 ) inputs and active HIGH ( Invert = 0 )inputs. // Inverted input should be 0 and 1 for test bench verification and hardware verification respectively. )( /* Port List of the N_Channel_Servo_Controller*/ input CLOCK , // The incoming clock is connected to this port. input RESET , // The reset pin is connected to this port. input LOAD_SIGNAL , // Latch the duty cycle of the PWM signal. input [(DUTY_CYCLE_WIDTH-1):0] DUTY_CYCLE_CONTROL, // Controls the duty cycle of the PWM signal, thus controlling the angle. input [(ADDRESS_WIDTH-1):0] SERVO_SELECTOR , // Determines the servo that has to be actuated. output [(NO_OF_CHANNEL-1):0] PWM_SIGNALS // The output of the module that is the input to the Servo motor. ); wire CLOCK_CONNECTOR; // Net to connect output of the frequency divider to the servo motor controller. reg INVERT_RESET; // Inverts the input reset signal for LOW active low inputs. reg INVERT_LOAD_SIGNAL; // Inverts the input load signal for LOW active low inputs. localparam HIGH_VALUE = 1; // Local parameter with value 1. reg [(NO_OF_CHANNEL-1):0]INDIVIDUAL_LOAD ; // Each channel has its individual load register. /* To check whether the inputs are active LOW or not and then take necessary actions */ always @ ( RESET , LOAD_SIGNAL ) begin // Always statement that changes with respect to reset and load signals. if ( INVERTED_INPUT ) // Check if the inputs are active LOW or active HIGH. begin // If yes, invert the inputs. INVERT_RESET <= ~ RESET; INVERT_LOAD_SIGNAL <= ~ LOAD_SIGNAL; end else begin // If no, assign the same inputs. INVERT_RESET <= RESET; INVERT_LOAD_SIGNAL <= LOAD_SIGNAL; end end /* Instantiating the frequency divider to convert incoming clock frequency to fixed frequency. */ Frequency_Divider # ( . INCOMING_CLOCK_FREQUENCY ( CLOCK_FREQUENCY ) ) Clock_Enable_Generator ( . FD_CLOCK_IN ( CLOCK ), . FD_RESET ( INVERT_RESET ), . FD_CLOCK_OUT ( CLOCK_CONNECTOR ) ); genvar COUNT; // Creating a general variable to implement a for loop. generate for ( COUNT = 0 ; COUNT < NO_OF_CHANNEL ; COUNT = (COUNT + 1) ) // Creating a for loop to instantiate N number of servomotors. begin : Servo_Motor_Driver_Block_loop localparam NUMBER = COUNT ; // A local parameter to hold the number of possible servo combinations. /* To check the value of the load signal and then take necessary actions. */ always @ ( INVERT_LOAD_SIGNAL ) begin if ( INVERT_LOAD_SIGNAL ) // Check whether the load signal button has been pressed. begin if ( SERVO_SELECTOR == NUMBER ) // Check if the input selector is same as that of the current servomotor ID. begin INDIVIDUAL_LOAD[COUNT] <= 1'b1; // Set the load of that particular servomotor to HIGH. (This signal should not remain always HIGH). end else begin INDIVIDUAL_LOAD[COUNT] <= 1'b0; // If the input selector does not match to the current servomotor ID, then set the load of the end // that particular servomotor to HIGH. end else begin INDIVIDUAL_LOAD[COUNT] <= 1'b0; // Once the load signal button is released set all the load signals to LOW. end end Servo_Motor_Controller Servo_Motor_Driver_Block ( // Instantiating N number of servomotors with separate PWM signal and load signal. . CLOCK_SIGNAL ( CLOCK_CONNECTOR ), . CLOCK_ENABLE ( HIGH_VALUE ), . SERVO_RESET ( INVERT_RESET ), . LOAD_SIGNAL ( INDIVIDUAL_LOAD[COUNT] ), . DUTY_CYCLE_CONTROL ( DUTY_CYCLE_CONTROL ), . PWM_SIGNAL ( PWM_SIGNALS [COUNT] ) ); end endgenerate endmodule // End of the module.

APPENDIX – B

Appendix B consists of all Verilog test bench files used in the project.

SECTION 1: Servo Motor Controller test bench

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : Servo_Motor_Controller_tb Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to control a single SG90 Servo Motor. The servo motor requires an input PWM signal to drive to the required angular position. The PWM signal for the SG90 motor should have a clock period of 20 ms. The desired angular position can be obtained by controlling the ON period (duty cycle) of the PWM signal. The duty cycle is can be manually varied by changing the 8 bit inputs connected to 8 different slide switches. The value of the duty cycle is latched to the output by using a load signal connected to a push button. The servo motor can be initialised to a default position by pressing the reset push button. */ //////////////////////////////////////////////////////////////////////////////// `timescale 1 ns /100 ps module Servo_Motor_Controller_tb; /* Parameter List of the Servo_Motor_Controller_tb */ localparam NUM_CYCLES = 50000; // Simulate this many clock cycles. Maximum value is 1 billion. localparam CLOCK_FREQ = 128000; // Current Clock frequency (in Hz). localparam RST_CYCLES = 2; // Number of cycles of reset at beginning. localparam REPEAT_DUTY_CYCLES = 25600; // Repeat values for different values of the input. localparam WAIT_PERIOD = 2; // Creating a small delay. /* Test Bench Generated Signals of the Servo_Motor_Controller_tb */ reg TB_CLOCK ; // Connects to the clock of the servo motor controller. reg TB_ENABLE ; // Connects to the enable of the servo motor controller. reg TB_RESET ; // Connects to the reset of the servo motor controller. reg TB_LOAD_SIGNAL ; // Connects to the load signal of the servo motor controller. reg [7:0] TB_DUTY_CYCLE_CONTROL ; // Connects to the 8-bit duty cycle control of the servo motor controller. /* Device Under Test (DUT) Output Signals of the Servo_Motor_Controller_tb */ wire TB_PWM_SIGNAL; /* Device Under Test (DUT) of the Servo_Motor_Controller_tb */ Servo_Motor_Controller Servo_Motor_Controller_DUT ( // Setting the connections to their corresponding ports. .CLOCK_SIGNAL ( TB_CLOCK ), .CLOCK_ENABLE ( TB_ENABLE ), .SERVO_RESET ( TB_RESET ), .LOAD_SIGNAL ( TB_LOAD_SIGNAL ), .DUTY_CYCLE_CONTROL ( TB_DUTY_CYCLE_CONTROL ), .PWM_SIGNAL ( TB_PWM_SIGNAL ) ); /* Reset the entire control system so that the servo initializes to the default value. */ initial begin TB_RESET = 1'b1; // Set the reset signal to HIGH. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH. repeat( RST_CYCLES ) @ ( posedge TB_CLOCK ); // Wait for a couple of clocks. TB_RESET = 1'b0; // Set the reset signal to LOW. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to LOW. end /* Clock generator and simulation time limit. */ initial begin TB_CLOCK = 1'b0; // Initialise the clock to zero. end real HALF_CLOCK_PERIOD = (1000000000.0 / $itor(CLOCK_FREQ)) / 2.0; // Calculating the time delay for each half of the clock cycle and storing it in a variable. integer half_cycles = 0; // Variable to count the elapsed number of half cycles. always begin TB_ENABLE = 1'b1; // Set the enable signal to HIGH. /* Duty Cycle Control = 0 */ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd0; // Assign a value of 0 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end /* Duty Cycle Control = 128 */ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd128; // Assign a value of 128 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end /* Duty Cycle Control = 255 */ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd255; // Assign a value of 255 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end $stop; // Break the simulation end endmodule

SECTION 2: Frequency divider test bench

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : Frequency_Divider_tb Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to reduce the incoming clock rate to a fixed value. The fixed value is 128 kHz and the incoming clock rate can vary from 128 kHz to 100 MHz. */ //////////////////////////////////////////////////////////////////////////////// `timescale 1 ns /100 ps module Frequency_Divider_tb; /* Parameter List of the Frequency_Divider_tb */ localparam NUM_CYCLES = 50000000; // Simulate this many clock cycles. Maximum value is 1 billion. localparam CLOCK_FREQ = 50000000; // Current Clock frequency (in Hz). localparam RST_CYCLES = 2; // Number of cycles of reset at beginning. /* Test Bench Generated Signals of the Frequency_Divider_tb */ reg TB_CLOCK_IN ; // Connects to the clock of the frequency divider circuit. reg TB_RESET ; // Connects to the reset of the frequency divider circuit. /* Device Under Test (DUT) Output Signals of the Frequency_Divider_tb */ wire TB_CLOCK_OUT; /* Device Under Test (DUT) of the Frequency_Divider_tb */ Frequency_Divider Frequency_Divider_DUT ( // Setting the connections to their corresponding ports. .FD_CLOCK_IN ( TB_CLOCK_IN ), .FD_RESET ( TB_RESET ), .FD_CLOCK_OUT ( TB_CLOCK_OUT ) ); /* Reset the entire control system so that the servo initializes to the default value. */ initial begin TB_RESET = 1'b1; // Set the reset signal to HIGH. repeat( RST_CYCLES ) @ ( posedge TB_CLOCK_IN ); // Wait for a couple of clocks. TB_RESET = 1'b0; // Set the reset signal to LOW. end /* Clock generator and simulation time limit. */ initial begin TB_CLOCK_IN = 1'b0; // Initialise the clock to zero. end real HALF_CLOCK_PERIOD = (1000000000.0 / $itor(CLOCK_FREQ)) / 2.0; // Calculating the time delay for each half of the clock cycle and storing it in a variable. integer half_cycles = 0; // Variable to count the elapsed number of half cycles. always begin /* Generating individual half cycles of clock. */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. /* Check if we have simulated enough half clock cycles. */ if (half_cycles == (2*NUM_CYCLES)) begin half_cycles = 0; // Reset half cycles, so if we resume running with "run -all", we perform another chunk. $stop; // Break the simulation end end endmodule

SECTION 3: Test single servomotor test bench

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : Test_Single_Servo_Motor_tb Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to control a single SG90 Servo Motor. The servo motor requires an input PWM signal to drive to the required angular position. The PWM signal for the SG90 motor should have a clock period of 20 ms. The desired angular position can be obtained by controlling the ON period (duty cycle) of the PWM signal. The duty cycle is can be manually varied by changing the 8 bit inputs connected to 8 different slide switches. The value of the duty cycle is latched to the output by using a load signal connected to a push button. The servo motor can be initialised to a default position by pressing the reset push button. */ //////////////////////////////////////////////////////////////////////////////// `timescale 1 ns /100 ps module Test_Single_Servo_Motor_tb; /* Parameter List of the Test_Single_Servo_Motor_tb */ localparam CLOCK_FREQ = 50000000; // Current Clock frequency (in Hz). localparam RST_CYCLES = 1; // Number of cycles of reset at beginning. localparam REPEAT_DUTY_CYCLES = 2000000; // Repeat values for different values of the input. localparam WAIT_PERIOD = 2; // Creating a repeat block to make the reset the load signal. /* Test Bench Generated Signals of the Test_Single_Servo_Motor_tb */ reg TB_CLOCK_IN ; // Connects to the clock of the servo motor controller. reg TB_RESET ; // Connects to the reset of the servo motor controller. reg TB_LOAD_SIGNAL ; // Connects to the load signal of the servo motor controller. reg [7:0] TB_DUTY_CYCLE_CONTROL ; // Connects to the 8-bit duty cycle control of the servo motor controller. /* Device Under Test (DUT) Output Signals of the Test_Single_Servo_Motor_tb */ wire TB_PWM_SIGNAL; // Connects to the PWM signal of the servomotor. wire TB_DIVIDED_CLOCK; // This signal is used only in test bench verification. /* Device Under Test (DUT) of the Test_Single_Servo_Motor_tb */ Test_Single_Servo_Motor Test_Single_Servo_Motor_DUT ( // Setting the connections to their corresponding ports. .CLOCK ( TB_CLOCK_IN ), .RESET ( TB_RESET ), .LOAD_SIGNAL ( TB_LOAD_SIGNAL ), .DUTY_CYCLE_CONTROL ( TB_DUTY_CYCLE_CONTROL ), .DIVIDED_CLOCK ( TB_DIVIDED_CLOCK ), // This signal is used only in test bench verification. .PWM_SIGNAL ( TB_PWM_SIGNAL ) ); /* Reset the entire control system so that the servo initializes to the default value. */ initial begin TB_RESET = 1'b1; // Set the reset signal to HIGH. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH. repeat( RST_CYCLES ) @ ( posedge TB_CLOCK_IN ); // Wait for a couple of clocks. TB_RESET = 1'b0; // Set the reset signal to LOW. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to LOW. end /* Clock generator and simulation time limit. */ initial begin TB_CLOCK_IN = 1'b0; // Initialise the clock to zero. end real HALF_CLOCK_PERIOD = (1000000000.0 / $itor(CLOCK_FREQ)) / 2.0; // Calculating the time delay for each half of the clock cycle and storing it in a variable. integer half_cycles = 0; // Variable to count the elapsed number of half cycles. always begin /* Duty Cycle Control = 0 */ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd0; // Assign a value of 0 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end //////////////////////////////////////////////////////////////////////// /* Duty Cycle Control = 128 */ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd128; // Assign a value of 0 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end //////////////////////////////////////////////////////////////////////// /* Duty Cycle Control = 255 */ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd255; // Assign a value of 0 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK_IN = ~ TB_CLOCK_IN; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end $stop; // Break the simulation end endmodule

SECTION 4: N-channel 8-bit servo controller test bench

//////////////////////////////////////////////////////////////////////////////// /* FPGA Project Name : N - Channel Servo Motor Controller Top level Entity Name : N_Channel_Servo_Controller_tb Target Device : Cyclone V Code Author : Shrajan Bhandary Date Created : 08/03/2019 Location : University of Leeds Module : ELEC5566M FPGA Design for System-on-chip ------------------------------------------------------------------------------- Description of the Verilog Module: The module is used to instantiate a parametrized number of servo controllers to allow the IP core to have N independent channels. This module has a clock input, an N-bit PWM output, a single 8-bit input to set the duty cycle, a 1-bit load signal to latch the duty cycle value, and an address input to control which servo is being updated. */ //////////////////////////////////////////////////////////////////////////////// `timescale 1 ns /100 ps module N_Channel_Servo_Controller_tb; /* ceil(log2(N)) Preprocessor Macro */ `define clog2(x) ( \ ((x) <= 2) ? 1 : \ ((x) <= 4) ? 2 : \ ((x) <= 8) ? 3 : \ ((x) <= 16) ? 4 : \ ((x) <= 32) ? 5 : \ ((x) <= 64) ? 6 : \ ((x) <= 128) ? 7 : \ ((x) <= 256) ? 8 : \ ((x) <= 512) ? 9 : \ ((x) <= 1024) ? 10 : \ ((x) <= 2048) ? 11 : \ ((x) <= 4096) ? 12 : 16) /* Parameter List of the N_Channel_Servo_Controller_tb */ localparam CLOCK_FREQ = 50000000 ; // Current Clock frequency (in Hz). localparam RST_CYCLES = 1 ; // Number of cycles of reset at beginning. localparam REPEAT_DUTY_CYCLES = 2000000 ; // Repeat values for different values of the input. localparam WAIT_PERIOD = 2 ; // Creating a repeat block to make the reset the load signal. localparam NO_OF_CHANNEL = 4 ; // The number of servos to be controlled corresponding to number of PWM signals. localparam ADDRESS_WIDTH = `clog2(NO_OF_CHANNEL); // Determine the required width to of the servo selector. /* Test Bench Generated Signals of the N_Channel_Servo_Controller_tb */ reg TB_CLOCK ; // Connects to the clock of the servo motor controller. reg TB_RESET ; // Connects to the reset of the servo motor controller. reg TB_LOAD_SIGNAL ; // Connects to the load signal of the servo motor controller. reg [7:0] TB_DUTY_CYCLE_CONTROL ; // Connects to the 8-bit duty cycle control of the servo motor controller. reg [(ADDRESS_WIDTH-1):0] TB_SERVO_SELECTOR ; // Connects to the N-bit servo selector of the servo motor controller. /* Device Under Test (DUT) Output Signals of the N_Channel_Servo_Controller_tb */ wire [(NO_OF_CHANNEL-1):0] TB_PWM_SIGNALS; // Connects to the N-bit PWM signal of the servomotors. /* Device Under Test (DUT) of the N_Channel_Servo_Controller_tb */ N_Channel_Servo_Controller N_Channel_Servo_Controller_DUT ( // Setting the connections to their corresponding ports. .CLOCK ( TB_CLOCK ), .RESET ( TB_RESET ), .LOAD_SIGNAL ( TB_LOAD_SIGNAL ), .DUTY_CYCLE_CONTROL ( TB_DUTY_CYCLE_CONTROL ), .SERVO_SELECTOR ( TB_SERVO_SELECTOR ), .PWM_SIGNALS ( TB_PWM_SIGNALS ) ); /* Reset the entire control system so that the servo initializes to the default value. */ initial begin TB_RESET = 1'b1; // Set the reset signal to HIGH. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH. repeat( RST_CYCLES ) @ ( posedge TB_CLOCK ); // Wait for a couple of clocks. TB_RESET = 1'b0; // Set the reset signal to LOW. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to LOW. end /* Clock generator and simulation time limit. */ initial begin TB_CLOCK = 1'b0; // Initialise the clock to zero. end real HALF_CLOCK_PERIOD = (1000000000.0 / $itor(CLOCK_FREQ)) / 2.0; // Calculating the time delay for each half of the clock cycle and storing it in a variable. integer half_cycles = 0; // Variable to count the elapsed number of half cycles. always begin /* Duty Cycle Control = 0 and servo selector = 0*/ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd0; // Assign a value of 0 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. TB_SERVO_SELECTOR = 2'd0; // Select the first servo. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end //////////////////////////////////////////////////////////////////////// /* Duty Cycle Control = 128 and servo selector = 1*/ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd128; // Assign a value of 128 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. TB_SERVO_SELECTOR = 2'd1; // Select the second servo. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end //////////////////////////////////////////////////////////////////////// /* Duty Cycle Control = 255 and servo selector = 2*/ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd255; // Assign a value of 255 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. TB_SERVO_SELECTOR = 2'd2; // Select the third servo. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end //////////////////////////////////////////////////////////////////////// /* Duty Cycle Control = 192 and servo selector = 3*/ repeat ( REPEAT_DUTY_CYCLES ) // Repeat the loop for some time. begin TB_DUTY_CYCLE_CONTROL = 8'd192; // Assign a value of 192 for the duty cycle control. /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. TB_SERVO_SELECTOR = 2'd3; // Select the fourth servo. TB_LOAD_SIGNAL = 1'b1; // Set the load signal to HIGH so that the output latches on to the required duty cycle. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. end repeat ( WAIT_PERIOD ) // Repeat the loop for some time. begin /* Generating individual half cycles of clock */ #(HALF_CLOCK_PERIOD); // Delay for half a clock period. TB_CLOCK = ~ TB_CLOCK; // Toggle the clock signal. half_cycles = half_cycles + 1; // Increment the count of number of half cycles. TB_LOAD_SIGNAL = 1'b0; // Set the load signal to HIGH so that the output latches on to the required duty cycle. end $stop; // Break the simulation end endmodule

APPENDIX – C

Appendix – C contains the hardware verification images.

SECTION 1: Single servo motor controller. The servomotor is at its default position of 0 degrees since the value has not been loaded.

SECTION 2: Single servo motor controller. The servomotor is at -79.46 degrees since the value of input duty cycle control = 15.

SECTION 3: Single servo motor controller. The servomotor is at -87.3 degrees since the value of input duty cycle control = 252.

SECTION 4: 3 – Channel servo motor controller. All three servomotors are at their default position of 0 degrees due to input servo selector value = 0 and input duty cycle control = 0.

SECTION 5: 3 – Channel servo motor controller. First servo on the left is at 10.54 degrees due to input servo selector value = 0 and input duty cycle control = 15. The remaining servomotors are at their default position of 0 degrees.

SECTION 6: 3 – Channel servo motor controller. First servo on the right is at 10.54 degrees due to input servo selector value = 1 and input duty cycle control = 15. The servomotor at the centre is its default position of 0 degrees.

SECTION 7: 3 – Channel servo motor controller. The servomotor at the centre is at 10.54 degrees due to input servo selector value = 2 and input duty cycle control = 15.